MCS7840/20 High-Speed USB to Multi Serial Device Driver

For Windows CE 7.0

Contents

1. Introduction	1

2. Resource Requirements	1

3. Install the Driver	1

4. Test the device	2

## 1. Introduction

This document provides the information about how to install the driver for ASIX MCS7840/7820 High-Speed USB to Multi Serial device on windows CE .NET 7.0 environments using the CAB file or integrating ASIX driver into NK.bin binary image.

## 2. Resource Requirements

For installation of the driver using .cab file, the following resources are needed:

- CEPC/Windows CE Embedded Device with Windows CE 7.0 OS

- PC having Win 2K/XP.

- MCS 7840/20 USBToSerial device.

- NULL MODEM Cable to test MCS7840 serial ports.

## 3. Install the Driver

Customer can use any one this method, he/she find applicable.

- Take a flash/pen drive copy the “SpecCe50.cab” file provided and install on required platform, MCS7840 is installed and ready to use on required platform.

- If you can use platform builder tools on required platform, make use of “Remote File Viewer” tool to import cab file on to platform and install it on device.

- If you want to integrate “SpecCe50.dll” in “NK.bin” image, please follow the below procedure:

Procedure to integrate MCS7840 driver into target platform:

Note: Make the “OSDesign” first with Zero Errors, then add the following changes to your platform.

1) Search file “platform. Bib” in BSP provided, open, and add the following lines to the file and save it:

MODULES

; Name                Path                                                                  Memory   Type

;  --------------  ---------------------------------------------  --------------------------------------

SpecCe50.DLL     $(_FLATRELEASEDIR)\ SpecCe50.DLL            NK            SHQ

2) Search “platform.reg” file in BSP, open, add the following lines to it and save it:

; ##########   Registry values for the MCS78xx USB to Multi Serial Driver ##########

; 7840; if user wants four serial ports [HKEY_LOCAL_MACHINE\Drivers\USB\LoadClients\38672_30784\255_0_255\255_0_255\MosHSUsbSerialClass]

"Dll"="SpecCe50.DLL"

; 7820; if user wants two serial ports [HKEY_LOCAL_MACHINE\Drivers\USB\LoadClients\38672_30752\255_0_255\255_0_255\MosHSUsbSerialClass]

"Dll"="SpecCe50.DLL"

; 7843; if user wants three serial ports   [HKEY_LOCAL_MACHINE\Drivers\USB\LoadClients\38672_30787\255_0_255\255_0_255\MosHSUsbSerialClass]

"Dll"="SpecCe50.DLL"

; 7810; if user wants one serial port   [HKEY_LOCAL_MACHINE\Drivers\USB\LoadClients\38672_30736\255_0_255\255_0_255\MosHSUsbSerialClass]

"Dll"="SpecCe50.DLL"

[HKEY_LOCAL_MACHINE\Drivers\USB\ClientDrivers\MosHSUsbSerialClass]

"Dll"="SpecCe50.DLL"

3) Copy and Paste the “SpecCe50.dll” provided into “RelDir” of build “OSDesign”

(Drive :> \WINCE600\OSDesigns\“OSDesign Name” \RelDir\CEPC_x86_Release)

Here in example we paste the mosusbce.dll into “OSDesign” for Release type on CEPC device.

4) Build the platform again as you did earlier, ensure zero errors.

5) From platform builder select “Build OS” -> “Make Run Time Image”.

6) Ensure zero errors; MCS7840 driver is ready for use on required platform.

Note: If you get error that “mosusbce.dll” file is not on disk, and then run these steps:

That is Step3 and   Step5 again.

## 4. Test the MCS7840/20 device

One can use “MCSTest.exe” application to test the installed MCS7840/20 serial ports on WinCE Device for loopback, send pattern and receive pattern tests.